Urban Institute

Purpose

This guide explains how to create the Study Area in a mapping software—in this case, QGIS. This allows you to create a shapefile of the corridor (i.e. the street(s) of focus) as well as the study area (i.e. the 0.5-mile area around the corridor). This also allows you to find the census tracts that intersect the study area, which is necessary for analyzing the population of the study area.

Instructions

- Open QGIS

  - New Empty Project

- Open Baselayer

  - Web → Quickmapservices → OSM Standard

  - If you don’t have the plugin, go to Plugins, look up QuickMapServices, and download it

  - Zoom into Los Angeles and set the EPSG to 2229 (for Southern California)

- Draw the corridor

  - Layer → Create Layer → New Shapefile Layer

    - Name file and save to proper folder

    - Geometry Type = Line String

    - Dimensions = EPSG 2229

  - Draw the street on the map

    - Select toggle editing (pencil icon)

    - Select add line feature (3 icons to the right of the pencil)

    - Left click on starting point

      - Left click throughout to change angle

    - Right click at end point

    - Save layer edits (icon to the right of pencil)

    - Make more visible

      - Right click the layer in the left-hand menu

      - Properties → simple line → increase stroke width

  - Export

- Add 0.5 mile buffer

  - Select the layer in left-hand menu

  - Vector → Geoprocessing tools → Buffer

    - Distance: 0.5 miles

    - Segments: 20

    - Run

  - Export (if file location not already set)

- Add all CA census tracts to the file

  - Layer → Add Layer → Add Vector Layer → select CA_census_Tracts file from the Data folder

  - Set CRS for CA_census_Tracts to EPSG 2229

    - Right-click the feature on the left-hand side → Export → Save Features as → update CRS to 2229

- Intersect census tracts with buffer

  - Vector → Geoprocessing Tools → Intersection

    - Input layer: Census tracts

    - Overlay layer: Buffer

    - Run

  - Export the intersection

- Make sure all exports are saved before closing out